Specifications

Standard Trucks:

Fabricated to AAR standards

Cast steel side frames and bolsters

Two,- four-wheel, swivel-type

56-1/2” - standard wheel gauge

33” multiple wear Steel carwheels

Forged steel axles

6-1/2” x 12 anti-friction roller bearing journals

7” x 12” Anti-friction roller bearing journals

(DE-600 & DE-650 only)

Bolster springs or rubber load suspension

Sliding wedges on top of side frames

Four, 7’ x 8” brake cylinders for 8-wheel braking

One, manual handbrake for 4 - wheel braking

Standard Traction Equipment:

Two, 115 horsepower traction motors

Enclosed travel gearing in oil bath lubrication

Quick disengaging travel arrangement

Gear ratios:

15.1:1 – Models DE 300S & DE 400

14.98:1 – Models DE 600 & DE 650

One traction generator coupled to engine flywheel

Clearance – top rail to bottom of gear case

4-3/4” – Models DE 300S & DE 400

4-1/16” – Models DE 600 & DE 650

Standard Carbody:

All welded, extra heavy-duty, steel construction

Anti-friction bearing rotating ring

AR approved hand grabs and foot steps

Anti-skid deck plates

Manual sliding beam type outriggers

Top operated type “E” couplers

Removable outrigger pockets

Standard Rotating Deck:

All welded, heavy-duty, steel construction

Fully counterweighted at factory (option)

200 gallon fuel tank with gauges

Functionally designed shafting

Arrangement for easy access to all shafting and   operating mechanism

6

Standard Cab:

Heavy gauge sheet metal panel-type construction

Interconnecting walkway between machine house and operators cab with insulated door

Operators cab is insulated with acoustical foam

Safety glass furnished throughout

Functionally located control equipment

Air operated horn

Windshield wiper and cab heater

20 lb Ansul fire extinguisher

Sliding front and side cab windows

Standard Cab:

24 volt, 115 ampere alternator

Two, - 150 watt flood lights on cab front

One, - 150 watt flood lights in boom

One, - 25 watt red tail light

One, - 50 watt light in operator cab

Four, - 50 watt lights in machinery house

One, - 24 volt receptacle in operator cab

Batteries: Two, heavy duty 12 volt connected in series providing necessary power

Standard Boom and Rigging:

50 ft. standard two piece Bolt connected boom.

5 ft. and 10 ft. center section available

Welded lattice-type construction of high strength low alloy chord angles with heavy duty pipe lacing

Anti-friction boom point sheaves:

Two - 30” sheaves:  Model DE-300S

Three - 30 sheaves:  Model DE-400

Five - 30 sheaves:  Model DE-600 & DE-650

Single, 7/8” Boom Hoist Cable:

8 parts:	  Models DE-300S & DE-400

12 parts:	  Models DE-600

Spring cushioned telescopic boom stops

Automatic / adjustable boom hoist limit switch

Main hoist cable:		7/8”

Auxiliary hoist cable:		7/8”

Standard Machinery:

Power Unit

Detroit Diesel: Series 60 is standard

Cummins and Caterpillar are optional

Indicator light for low oil pressure & high water temperature